H

A

N

G

I

N

G

R O C K

RECREATION RESERVE

M A N A G E M E N T

P L A N

A Report to the Committee

of Management by the

LODER & BAYLY

Consulting Group

September 1993

CONTENTS

Page

Table of Contents

List of Figures

1.

2.

3.

4.

5.

6.

7.

8.

9.

10.

11.

12.

13.

14.

INTRODUCTION

LOCATION & CONTEXT

PHYSICAL DESCRIPTION

HISTORY

SITE ANALYSIS

OPPORTUNITIES AND CONSTRAINTS

VISITOR SURVEY

LANDSCAPE MANAGEMENT UNITS

ISSUES IDENTIFICATION

SITE ACCESS & CIRCULATION

REVENUE COLLECTION AND ACCESS

MANAGEMENT PLAN RECOMMENDATIONS

STAGING OF WORKS/PRIORITY OF ACTIONS

STANDARD DETAILS

3

5

10

12

15

20

25

31

34

41

43

61

63

i

ii

APPENDIX

FLORA AND FAUNA

REVENUE COLLECTION AND ACCESS

WEED CONTROL IN NATURAL BUSHLAND AREAS

CURRENT RESERVE REGULATIONS

CURRENT RESERVE FIRE MANAGEMENT PLAN

POSSIBLE RECREATIONAL/OPPORTUNITIES

VISITOR SURVEY FORM

REFERENCES

i

LIST OF FIGURES

Page

SITE LOCATION

REGIONAL CONTEXT

VEGETATION

VISUAL ANALYSIS

SITE OPPORTUNITIES

SITE ISSUES AND CONSTRAINTS

LANDSCAPE MANAGEMENT UNITS

EXISTING CIRCULATION AND PARKING

MAJOR EVENTS CARPARKING

ACCESS AND CIRCULATION

ROAD CLOSURE BEHIND RACE ENCLOSURE

SW. PICNIC AREA

EASTERN PICNIC AREA

RACE DAY VIEWING

ACCESS TO ROCK

MASTERPLAN

STANDARD DETAILS

2

4

8

14

18

19

29

33

34

35

36

37

38

39

40

44

from 63 onwards

ii

1.

INTRODUCTION

The Loder & Bayly Consulting Group were commissioned by the

Hanging Rock Recreation Reserve Committee of Management to

prepare a Landscape Management Plan for the Hanging Rock

Recreation Reserve.

The timing of this study is opportune, as it co-incides with the

realization of a long term goal of the Committee of Management to

purchase the adjacent property of 22 hectares to the east of the

Reserve and incorporate this land into the existing Hanging Rock

Recreation Reserve. The new land acquisition is a real asset, as it

facilitates the easier implementation of the main study goal as

recommended by the Land Conservation Council in its review of

District 1 of the Melbourne Area:-

" That the area continue to be used for the purposes approved by

Government following publication of the final recommendation for

the Melbourne area in 1977.

Notes:

1.

The existing recreation reserve, Parish of Newham,

encompasses Hanging Rock. The significant geological,

scenic, historical and floristic values of Hanging Rock

should be protected. "

It does this by allowing greater scopes for the dispersal of activities

which are currently concentrated close to the base of the rock,

exerting pressure on the values requiring protection.

In addition to the protection of natural values, major historical values

to be preserved are the bi-annual picnic race meetings which have

been run for over 100 years on the site and are of significant tourism

and cultural value.

The study brief aims to achieve a balance between natural and cultural

values so that both are enhanced by implementation of the final

management plan which will be developed following a public review

phase.

1

2.

LOCATION AND CONTEXT

Hanging Rock Recreation Reserve is located in a rural farmland

setting approximately 80km N.N.W of Melbourne, generally one hour

drive. It is 7km E.N.E. of Woodend, accessed by the Calder Highway

and roughly half way between Melbourne and Bendigo.

To the south of the Rock, the Macedon Ranges run East to West, with

their most prominent points, the Camel's Hump and Mt. Macedon

easily seen from the Rock Reserve. To the north-east the Cobaw

Range runs parallel with the Macedon Ranges, providing a forested

background to the rural areas surrounding the Rock.

The surrounding area is mainly cleared, undulating grazing land with

some cropping. Being reasonably close to Melbourne, the area has

seen an increase in commuter residents and the resultant growth in

hobby farming and small rural residential properties.

The Victorian Tourism Commission and the local tourist authorities

heavily promote the area for day tourists, in conjunction with Mt.

Macedon, (although this has been far less of an emphasis after the

Ash Wednesday bushfires) Gisborne, Woodend, Kyneton etc. The

growth of a high quality wine industry is likely to attract many more

tourists to the district.

3

3.

PHYSICAL DESCRIPTION

3.1

THE SITE

The Hanging Rock Recreation Reserve is now comprised of two

distinct areas. The first area, the original reserve, is 66 hectares in area

and is comprised of the Rock, racecourse and picnic areas. The

second area of 22ha is the newly acquired property to the east which

is almost entirely treeless, having been used for grazing purposes.

The Rock and its surrounding bushland is located near the

north-western boundaries of the reserve, the rock itself covering

approx. 9 hectares and its summit of 711 m A.H.D. jutting 100 metres

above the surrounding plain.

Five mile creek, a tributary of the Campaspe River, flows from east to

west through the southern portion of the site, filling 3 dams in the

process. Little more than an open channel drain in its central stretch,

the creek stops flowing in the drier months. A water reserve at the

point where the creek enters the property from the east has more of an

overgrown/semi-natural feel.

The racetrack, located in the north-eastern corner close to and parallel

to the eastern boundary, takes up a very large proportion of the

reserve.

Detail about specific site characteristics follows.

3.2

SOILS AND GEOLOGY

Hanging Rock is a 100 metre high volcanic mamelon lying on the

time boundary between the older volcanics and a newer series. It is

part of an area with affinities with other volcanic provinces in N.S.W.

and Queensland and it has resulted from a process of Lava

differentiation that is restricted to two small areas of Victoria, those

two being Gisboume/Woodend and Daylesford.

On eruption approximately 6 million years ago, small volumes of

silica rich soda-trachyte lava were produced. Hanging Rock is the

only recorded location of this rock in Australia. The Lava was of a

higher viscosity than most, and the greater resistance to flow caused a

dome to form around the point of eruption, on cooling producing a

steep

5

sided, fractured mass which has been exaggerated by weathering to

produce the rock as we know it today.

During the initial weathering process, a solution rose to the surface to

form an erosion resistant crust. Where the crust has been disturbed,

the erosion process has been accelerated. As a generalisation sods are

deepest where the vegetation is most substantial, these areas being the

central area forest and around the base of the Rock.

The local soils are moderately fertile with those derived from the

soda-trachyte being less so with lower levels of Phosphorus and

Calcium.

3.3

CLIMATE

The climate is fairly typical of inland cool temperate areas with cool

wet winters and warm/hot dry summers. Climate data for Kyneton, 14

kms to the north-west follows:

RAINFALL

Mean monthly

-

Highest

Lowest

-

-

Jun 90mm

Jan 38mm

TEMPERATURE

Mean max

Mean min

3.4

-

-

-

-

Lowest

Highest

Lowest

Highest

-

-

-

-

Jul 10°C

Jan 26°C

Jul 20°C

Feb 10.5°C

VEGETATION

The vegetation of the Hanging Rock Reserve constitutes one of the

few remnant pockets on the plain between the Macedon and Cobaw

Ranges and it is of great importance as it forms part of a movement

corridor for indigenous fauna.

The least disturbed parts of the site are the foot slopes to the Rock,

and the native grasslands inside the racecourse. The grassland is in

good condition and is floristically the most important plant

community in the reserve. The next least disturbed area are the

boundaries to the north-east and south-west. The remainder of the site

is heavily disturbed, particularly the area each side of the entry drive

and the

6

racecourse. The newly acquired property is an almost completely

treeless field of long pasture grass.

The Rock vegetation is typified by scattered and slightly stunted trees

and shrubs growing between the boulders, with the central forest area

with its deeper soils and protected aspect being an exception to this.

The upper slopes of the Rock are highly exposed, dry and the soils

shallow. Species found here include Eucalyptus pauciflora, and

Eucalyptus rubida, Exocarpus cuppressiformis and Acacia dealbata.

The mid slopes and central forest area with their deeper sods and

increased run-off retention are able to promote a greater variety and

density of vegetation. Species found here include Eucalyptus rubida,

E. Radiata, Acacia dealbata, A. melanoxylon, Exocarpus

cuppressiformis and Pomaderris aspera. The upper limit of bracken,

Pteridium esculentum is found here.

The footslopes and flats around the rock are more fertile, have a

greater depth of soil and higher and more consistent moisture levels.

The density of vegetation is greater on the south-side of the rock than

the north side due to it being shaded and therefore moister, although

the vegetation variety is greater on the north side where more annual

species are encountered especially in spring and summer. Typical

species include: Eucalyptus radiata, E. rubida, E. ovata, E. obliqua, E.

viminalis, Acacia dealbata, A. melanoxylon, A. mearnsii, Pomaderris

aspera, and Cassinia s.p. Bracken is at its densest around the base of

the rock and on the lower slopes.

The grassland to the north of the centre of the race track is an example

of remnant native grassland that would have existed in the area prior

to European settlement. Major species present include Themeda

triandra and Danthonia sp. This grassland is worthy of protection due

to its relative scarcity and is considered to be of national significance.

The main southern entry drive is lined by an avenue of exotic tree

species – Cupressus macrocarpa "lambertiana" aurea and Populus

canescens.

The main carpark has a variety of Eualyptus species planted in rows

between the bays, some of these being: Eucalyptus globulus, E.

cinerea, Acacia bailyana.

Recent plantings in the area of the creek flat have tended to include a

greater use of indigenous species.

A list of flora is included in the appendix.

7

3.5

FAUNA

For such a relatively small area, and one that is subject to such visitor

pressure, the Reserve is a well endowed with wildlife. The most

notable, and the one most sought by tourists, is the Koala, a dozen of

which live in the more heavily vegetated and undisturbed areas.

Released Wallabys used to live within the reserve but these were

attacked and killed by dogs prior to the current protective fence being

erected.

The reserve is valuable as a wildlife corridor, as it forms part of a link

between the Macedon and Cobaw Ranges and is a refuge for species

such as the Black Wallaby. It is also important as a transit area for

migratory birds.

The dam in the centre of the racecourse and the nearby mature and

dead trees form an important bird habitat area.

A list of fauna present in the reserve is included in the appendix.

9

4.

HISTORY

ABORIGINAL

Hanging Rock is located on what was the north western boundary of

the Wurundjeri tribal area. It does not appear to have had any spiritual

significance other than it being a "special" place for the Wurundjeri,

but any future development should be undertaken in consultation with

the Victorian Archaeological Survey.

Archaeological surveys, particularly the discovery of Keilor Man,

date an established human presence in the approximate area 36,000

years ago.

EUROPEAN

In 1836 Major Thomas Livingston Mitchell on a journey that would

lead to the opening up of the Port Phillip District gave the Rock its

official name, Mount Diogenes, in keeping with historical Greek

names he gave other areas in the district.

In 1837, settler Edward Dryden established a squatters run in the area

and for a while "the rock" was named Dryden's Rock after him. 1857

saw a high demand for land in the area with the larger squatter's runs

being sub-divided into smaller farms. William Adams purchased the

farm surrounding the Rock and, realizing its potential as a tourist

attraction, set about developing it as a resort. A dam across Five Mile

Creek created a lake of about a hectare in area which was stocked

with swans and other waterbirds. Exotic plants were introduced to

give the area a European flavour.

A carriage track was constructed running to the flat area above the

Southern line of pinnacles and in 1877 a racecourse was laid out on

the western side of the rock.

In 1884 the property was purchased by the Victorian Government,

added to an existing water reserve to the east of the Rock and named

Hanging Rock Recreation Reserve. The Reserve was then placed

under the control of the local Council as Committee of Management

appointed by the Minister for Lands.

In 1885 the Hanging Rock race club was formed and the racecourse

laid out in its current location in 1909.

10

The Rock is said to have been used as a look-out by at least two

Bushrangers, McDonald and the more infamous Dan "Mad Dog"

Morgan.

"Hanging Rock" was a name given to a formation over the path at the

top of the stairway where a large boulder is wedged between two

vertical columns.

The Reserve was managed by the Council of Newham and Woodend

until 1959 when three members of the Hanging Rock Race Club were

added to the Committee of Management. The management of the

reserve reverted to council in 1990.

Joan Lindsay's book "Picnic at Hanging Rock" published in 1967 and

the film based on the book, first screened in 1975 both heavily

publicized the Rock and led to increased visitation.

11

5.

SITE ANALYSIS

5.1

VIEWS TO THE ROCK

The first glimpses the tourist gains of "the Rock" are from the

distance on the approach roads from Woodend, Newham and

Macedon. The closer one gets to the entry off South Rock Road, less

of "the Rock" is visible as it is hidden by the concentration of

vegetation in the immediate vicinity. Once within the boundaries of

the reserve, the best views are from the small hill to the east, located

on the recently purchased property, where the Rock can be seen rising

above its skirting vegetation. There are also excellent views from this

hill down the valley toward Woodend.

5.2

VIEWS FROM THE ROCK

Given its elevation of some 100 metres above the surrounding plain,

the views are, as could be expected, quite superb, being spoilt only by

the short distance views to the west of the rural residential subdivision

at the foot of "the Rock".

To the south is the Camel's Hump and the Macedon Ranges, to the

North, the Jim Jim and Cobaw Range. From the eastern edge of the

summit, the views over the reserve and racecourse are breath taking,

with the racecourse buildings almost entirely hidden from view at the

foot of "the Rock".

5.3

INTERNAL VIEWS

The semi-vegetated riverflats form a pleasant backdrop to views from

the entry road, particularly where they form a partial visual enclosure

to the play meadows.

The actual creek is unattractive, drain-like and weed infested,

particularly in the section to the south of the racetrack.

Carparking areas to the south of the rock are bare, eroded and visually

intrusive. The access road to these carparks and the connection to the

northern picnic areas crowd the bottom of the rock intruding on the

natural resources.

12

Many structures over the site, particularly the toilet blocks and

corrugated fences and iron sheds of the racecourse, detract from the

visitor experience and should be removed or renovated.

13

6.

OPPORTUNITIES AND

CONSTRAINTS

The preceding analysis of the sites content, history and natural,

physical and perceptual components reveals the various opportunities

and constraints pertaining to the site in terms of its future use and

management. The following tables outline the sites opportunities and

constraints:

15

OPPORTUNITIES AND CONSTRAINTS

CONSTRAINTSPast zoning has allowed development at an undesirable

density to encroach on site - minimal or no buffer.

Contextual

Historical



Historical use of the site by the race club has meant thatmuch of the site has been alienated.

Physical





Current rock access system is congested on busy days.Creek flat area is flood prone with a low carryingcapacity in winter.Race track buildings enclosure and track fence alienatelarge areas of land from public.

Physicalcontinued

OPPORTUNITIESA resource of national geological significance in such close

proximity to Melbourne.

Within a short distance to other tourism features in the area

e.g. Mt. Macedon.

Current zoning will ensure rural setting remains intact.Opportunity to promote to tourists based on past cultural

events

such as picnic race meeting and the book and film "Picnic at

Hanging Rock".

Opportunity to provide interpretation based on geological

and cultural history.

Purchase of "22 ha" allows significant restructuring to take

away picnicking pressure from close to rock and morenatural environments.

Surrounding road system could double as the reserves

circulation system.

Opportunity for increased use of existing racecourse

facilities by public.

Kiosk located in prime position for up-grading.Toilet blocks are structurally sound and could be retained

and renovated.

Dam in centre of race track could be developed with board

walks and jetties.











Shape of race track blocks movement into new propertyfrom west to east.Race track location squeezes movement north-southbetween itself and base of rock.New property virtually treeless and won't be suitable forrelocating picnic activities for a considerable time.Current access fee collection system means Ranger isinhibited from undertaking other tasks.Roads outside kiosk make it a user "unfriendly" location.

Natural

OPPORTUNITIESOpportunity to increase habitat value of reserve by linking

bushland from Rock to surrounding area, making it moreinviting to roosting birds etc.

The race track has protected a natural grassland area in the

centre from trampling.

The creek has the potential to be re-shaped with ponds and

wetlands along it.

Less disturbed parts of the reserves vegetation may be

suitably restored as quality bushland.

Regular burning has aided Themeda grassland regeneration

Perceptual



Hill on new property has excellent views of rock and has thepotential to be developed as an activity node.

CONSTRAINTSCurrent rock climbing access to the base of rock iscausing damage to vegetation.Rock climbing may disturb nesting birds e.g. Wedge-Tail

Eagle

Riverflat area poorly drained.Dam in middle of race track used for irrigating track - not

a source of permanent water for fauna.

High numbers of tourists can create erosion on highly

erodible rock summit and sideslopes.

Quality of water in creek is uncertain due to possible

effluent run-off from septic tanks on site.

Weeds are a problem along creek.Creek is moderately erodedLack of creek flow over summer.Rigorous fire prevention measures such as burning-off

and fire tracks impact on habitat and flora regeneration.

Boundary roads to north-east, east and south are visually

intrusive.

Large numbers of visitors using Rock access stairs cause

congestion and therefore loss of experience.

7.

VISITOR SURVEY

7.1

INTRODUCTION

The survey was conducted for 7 days, commencing on Saturday 18th

January 1992 and finishing on Friday 24th January 1992 (just before

the Australia Day Races).

Questionnaires were handed to drivers of vehicles by the park ranger,

as the vehicles entered the park. One questionnaire was handed to

drivers of buses to fill out on behalf of the passengers.

Visitors were asked to complete the questionnaire and place it in one

of the boxes provided before they left the park that day.

Posters at strategic locations in the park reminded visitors to complete

the questionnaire.

Several hundred questionnaires were handed out and a total of 238

usable questionnaires were returned. A copy of the questionnaire is

included in the Appendix.

As it was entirely at the discretion of the visitor whether or not they

returned the questionnaire, the responses obtained cannot be regarded

as statistically representative of the total visitor population. They

provide instead an indicative picture of the type of visitor, why they

come to the park what they do there and improvements they would

like to see.

7.2



ANALYSIS OF RESULTS

The weekend is the most popular time to visit the park

(despite the fact that the survey took place in school

holidays).

Monday

Tuesday

Wednesday

Thursday

Friday

Saturday

Sunday

(No reply by 16)

14

16

13

18

45

55

61

replies

"

"

"

"

"

"

20



Cars represented the most frequent form of travel to the park,

although 10 buses were recorded in the sample.

Car

Bus

Bicycle

Other/No answer

224

10

2

2

(94%)

(4%)

The cars accounted for 1,027 visitors – this represents an average car

occupancy rate of 4.5 people. The 10 buses accounted for 164 people,

an average of 16 per bus, although actual numbers ranged from less

than 10 to 30 people per bus.



A large spread of ages was contained in the sample, as

follows:

0-4years

5-17"

18-24"

25-45"

46-60"

Over 60"

48

320

165

403

154

118

(4%)

(26%)

(14%)

(33%)

(13%)

(10%)

Visitors by bus tended to be older, on average, with a significant

number being over 60.

0- 4years

5-17"

18-25"

26-45"

46-60"

Over 60"



1

30

15

18

36

64

(1%)

(18%)

(9%)

(11%)

(22%)

(39%)

A large number of visitors by car and by bus came from both

overseas and other parts of Australia.

Came by car/bicycle from:

Local area

Melbourne Metro area

United Kingdom

Other overseas

Sydney

Other NSW

Brisbane

Other Qld.

Tasmania

34

104

20

3

5

6

5

5

6

(15%)

(46%)

(9%)

( 1%)

(2%)

(2%)

(2%)

(2%)

(2%)

21

Adelaide

Other S. A.

Perth/Western Australia

Bendigo/Ballarat

Other Victorian

Other Australian

7

4

9

7

11

2

(3%)

(2%)

(4%)

(3%)

(5%)

( 1%)

Note: Some interstate/overseas visitors were brought by

Melbourne residents.

Came by bus from:

Europe/America/Australia

Darwin/England

Australia Wide

Adelaide

Perth

Melbourne Metro area

2

1

2

1

1

3

Most people (other than those living locally) were visiting the

area as part of a day trip or passing through the area as part of

an extended trip.

On a day trip

Staying in local area

Passing through as part of an extended trip

152

38

48

(64%)

(16%)

(20%)

About half the sample were spending less than half a day at

the reserve – the same was true for bus visitors.

Spending all/most of day at reserve

Spending half a day at reserve

Spending less than half a day at reserve

35

83

119

(15%)

(35%)

(50%)

Many different activities were undertaken during the visit to

the Reserve - with a large percentage of the sample going up

the rock.

Climbing/going up Rock

Walking around

Sightseeing

Picnicking/BBQ

Relaxing

Social- with family/friends

Nature study

Informal games/ball games

Formal sport

198

152

147

125

112

69

34

22

2

(83%)

(63%)

(62%)

(53%)

(47%)

(29%)

(14%)

(9%)

1%)

22

Note: Multiple answers.



Clearly the desire to climb and/or see the rock and visit the

area made famous by the book and the film are the main

reasons for coming to Hanging Rock Reserve (rather than

some other place). Main reasons are shown in detail below:

Main Reasons for Coming To-day

To see/Climb rock/Hanging Rock38

The film/the movie/to find Miranda,32

"A Picnic at Hanging Rock"/The Book

Recreational activity/something to do/ 29

For enjoyment/Sightseeing

Never been before21

To bring overseas visitor/interstate20

visitor/tourists

Natural bush setting/beautiful spot12

On route back to Melbourne/on tour/11

on way home

Read/heard a lot about it/famous place 10

Place for Picnic/BBQ9

A place to walk and climb/exercise7

Family outing/group outing/nice setting 7

for family outing

Live locally/not far from home6

A days outing8

Close to Melbourne6

Recommended by a friend5

Been before and liked it5

Historical area/part of Australian history 4

Mystery/Adventure3

Other comments17

No answer29

Note: Multiple answers



On ways to improve the area, nearly half the sample had

either no opinion, or felt that the area was good as it is. The

entry fee was mentioned as a problem by a small number of

people.

Improvements to the walking trails, the map/pamphlet and

naming of significant areas on the rock were among some of

the constructive suggestions made. Others were keen to see

the area kept natural and revegetated with native plants, or

native animals and birds encouraged back to the area. The

table below shows details.

23

(16%)

(14%)

(12%)

(9%)

(8%)

(5%)

(5%)

(4%)

(4%)

(3%)

(3%)

(3%)

(3%)

(3%)

(2%)

(2%)

(2%)

(2%)

(7%)

1%)

How to Improve the Area

Don't change it/Nice as it is

No answer

Better walking tracks/less difficult

walking tracks/better signposted

tracks/walking distances and

difficulty defined/colour coded

walking tracks

Reduce/remove entry fee

Naming of locations at sites on rock/

naming locations referred to in

film/book

Better litter control ban bottles

on rock/more bins

Keep it natural/don't develop

More native animals/birds/sanctuary

for native animals/birds/

bring back koalas

Get rid of flies

Childrens play equipment/more for kids

Better pamphlets/more information on

pamphlets/on map

Get rid of ugly sheds/ugly toilets/

ugly wire fences

More BBQs/Electric BBQ's

Improve toilets

Handrails for older/disabled people

Lift/chairlift up rock

Maintenance of existing paths, handrail,

fences/tables

Get rid of Racecourse

Revegetate with native trees/plants/

plant native trees in picnic area

More information about history of the area/

aboriginal history

Swimming area

Refreshments/water at top

Improve area for tourists

Rock impressive but surroundings devastated

Information leaflets on arrival/map on arrival

Modernise/clean-up the shop/better food shop

Other comments

30 (13%)

85 (36%)

20 (8%)

18 (8%)

13 (5%)

11 (4%)

11 (5%)

10 (4%)

6

7

7

6

5

5

5

4

4

4

4

3

3

3

2

2

2

4

10

(3%)

(3%)

(3%)

(3%)

(2%)

(2%)

(2%)

(2%)

(2%)

(2%)

(2%)

( 1%)

(1%)

(1%)

(1%)

(1%)

(1%)

(2%)

(4%)

24

8.

LANDSCAPE MANAGEMENT

UNITS

A site such as the Hanging Rock Recreation Reserve contains a

variety of landscape types and settings, each of which are quite

individual in the way in which they respond to public use and in the

maintenance they require. It is necessary therefore to break the site up

into landscape management units. The objectives for management and

the proposed management actions for each are set out in the following

table as well as a Landuse Strategy/Planning Matrix.

25

OBJECTIVES AND ASSOCIATED ACTIONS FOR LANDSCAPE MANAGEMENT UNITS

PRESCRIBED ACTIONS

VEGETATION/RESTORATION



Remove all weeds and allow natural

regeneration.

Supplement with indigenous specieswhere re-generation will not take placenaturally.

Adopt measures to control future weed

invasion.

Retain dead trees as habitat value

except where of danger to public.

Remove dead or dangerous tree limbs.

Remove weeds along creekline.

Restore indigenous vegetation along

creeklines and around boundaries.

Revegetate picnic area and carpark on

new property.

Picnic areas to be predominately of

open upperstorey with grassedunderstorey regularly mown to 25-50mm.

Adopt weed control measures

appropriate to the specific problemspecies.





Sealed access road with turn

loops at ends. 4m wide for 1way. 6m wide for 2 way roads.

Sealed or gravelled 90° off

road parking bays grouped in 3-4s.





No major concentrated

carparks.

Removable bollards or

barriers. Allow access for majorevent overflow parking.



Park in bays and walk to picnic areas.Vehicles contained by marked bays andvehicle barriers/bollards.







VISITOR MANAGEMENT

Discourage people from using area except

where formed paths are present.

Interpretive trail will aid in directing

pedestrian flow.

BROAD MANAGEMENT

OBJECTIVES

1 Natural Dominant

EROSION CONTROL



VEHICLE ACCESS AND

CARPARKING



None — all located outside of

perimeter,

To maintain and manage as anatural bushland habit andthreatened species habitat area















Fencing is required in somearea to limit access and controldamage. Allow naturalregeneration. Construction ofstairway up rock may berequired in places.

2

Semi-Natural



To provide for passive recreationin an open woodland setting and torestore and rehabilitate creek edgeand surrounds. Will also act as abuffer to the threatened specieshabitat area.

Re-profile Creek banks andstabilize with planting andlocal rock where appropriate.



PRESCRIBED ACTIONS

VEGETATION/RESTORATION



Ensure vehicles don't compact soil

around existing trees by employingappropriate surfacing techniques.







Slowly replace invasive non-

indigenous trees with indigenous,

Only carpark where buses areallowed to park





VISITOR MANAGEMENT

All large buses (not mini-buses) must park here.

Due to formalised nature of area with hardsurfacing, little management will be required.

Signage required to direct to rock and picnicking.

Formalise and harden landscape treatments tocreek edge for increased pressure of use.

BROAD MANAGEMENT

OBJECTIVES

3 Intensive Use Picnic

EROSION CONTROL



To maintain and manage as a highcapacity carpark and centralactivity node.





Indigenous middle and lower

storey plants should be used whereappropriate around carparks andbuildings.

Formalise edging of

carparking areas and sealsurface to reduce dust.

VEHICLE ACCESS AND

CARPARKING



Sealed major entry road andsealed major carpark locatedopposite kiosk.



Construct defined paths to

heavily trafficked areas suchas kiosk and start of RockWalk.







Vehicle access to stables andcourse facilities allowed onlyon race days.

Road open to managementvehicles only and walkers forthe remainder of the year.

Major events parking allowedon oval.





Formalise drainage of

carpark and picnic areas toreduce damage to grasssurface.

Retain Poplars, Cypress Oaks and

Non- indigenous natives.



Allow selective regeneration ofappropriate large tree species thatwill develop large, clean trunks andstill allow views across race track



Race track to have an even grasscover aided by supplementaryirrigation in summer,





4

Racecourse

Picnic areas within race enclosure are to beopened up to general public use - walk in only.

To maintain and manage as a racecourse, for periodic events, casualpicnicking, and the oval for yearround sporting use.

5



To investigate the possibility ofregenerating indigenous grasslandspecies.

To maintain a cut or grazedheight of 200mm.

To reduce cut height in summer to100mm to reduce fire risk.

In the short to medium term fencewith electric fencing and allowcattle to graze on a seasonal basis.

Open Grassland



Oval lawn to be kept cut short.To remove invasive weed species.

Do nothing to encourage use, but don't stop publicaccess.

To maintain and manage asinformal grassland.







PRESCRIBED ACTIONS

VEGETATION/RESTORATION



Remove all weeds and debrisand manage to control futureinvasion,

Allow selected larger treespecies to regenerate which haveclean hunks and allow viewsacross track.

Conduct mosaic burns ofgrassland on a 5 yearly rotationto reduce biomass, increase internodal spaces and encourageregeneration.

Implement an ongoingmonitoring program.

Replant tree and shrub speciesusing appropriate weed/grasssuppressing matting.

Fence with electric fence toexclude cattle. (In short term)

Once upper storey is establishedstart to develop a lower storey.

Once established employ a 6-10year rotation mosaic burn.



No vehicle access allowed.



Public access not encouraged.



VEHICLE ACCESS AND

CARPARKING



No vehicle access allowed.

VISITOR MANAGEMENT

BROAD MANAGEMENTOBJECTIVES6 Open Native Grassland

EROSION CONTROL

To restore and manage as anindigenous grassland.



Current location of race course running raildiscourages public access. No special managementrequired to correct this. Trampling and rubbishdumping and burning to be avoided.







7

Bush Regeneration

To restore and manage as aeucalypt woodland withshrubby understorey.







LANDUSE STRATEGY / PLANNING MATRIX

NATURAL DOMINANT & OPEN

SPACE GRASSLAND

some habitat rehabilitation plus changes toincrease carry capacity - essentially amodified ground plane

common but merged with the landscape

all modes to trail heads plus extensive trailnetworks

minor level

high probability of social interaction;occasional peak time overloading

conspicuous control to protectconservation values

SEMI-NATURAL & BUSH

REGENERATION

INTENSE USE PICNIC &

RACECOURSE

some habitat rehabilitation plus significantchanges to maximise carrying capacity

very common and conspicuous

all modes

extensive

other people always present; high level ofsocial interaction; likely peak timeoverloading

possible 24 hour access and control mostlyrelated to health and safety

LandscapeChanges

habitat rehabilitation plus controlledaccess/management devices

Structures

signage & staircases only

Access

pedestrian, education and managementonly

Facilities

NIL

Social

low level contact and managed capacity

Control

controlled access and maximum habitatmanagement

9.

ISSUES IDENTIFICATION

The issues of primary concern in the formulation of an appropriate

management plan for the reserve are listed on the following page.

One of the key issues of Reserve Status, i.e. does it serve local, district

or regional population, has been established by anecdotal evidence as

well as being confirmed by the results of the Visitor Survey. The other

key issue is the purpose of the Reserve, i.e. will the reserve be used

for passive or active recreational pursuits. This has been confirmed by

the L. C. C. Melbourne Area Final Recommendations, that the reserve

is a recreation reserve but that its significant natural values be

protected. In other words, the right of the Hanging Rock Racing Club

to conduct race meetings at the reserve is acknowledged, but the

environment of the rock, its flora, fauna and geological features, must

be protected.

The following chart sets out the issues identification process:

31

ISSUES

ACCESSIBILITY AND CIRCULATION



Control of pedestrian movement around rock.









Weed control





Conflict between rock climbers, and thegeneral public and vegetation and faunaespecially avi-fauna.

Standard design details required for parkfurniture.

Upgrading of site structure, particularly toiletblocks.

Freedom of movement by sporting clubmembers to Tennis and Cricket facilities.

Best use of kiosk and vacant Rangersresidence.

Upgrading of Picnic facilities.

Provision of Playground equipment tocomply with Draft Australian Standards.

Upgrading of creek.

RECREATION ACTIVITIES/FACILITIES

RESTORATION OF LANDSCAPE



Removal or upgrading of existing access roads.



Location of new access roads.





Fee collection and number of entries.



Conflict between fuel reductions, burns/firebreaks and site vegetation restoration.

Should new property be fenced like existing?

Short term management of new property

--

RevegetationGrazing to reduce fire risk

Upgrading of existing carparks and standards fornew ones.











Upgrading of signage - need for standardised sign styles.



Increased area of race day parking spaces.



Any limit to numbers of people on site?



Erosion control

- on rock- high use picnic areas- creekline

10. SITE CIRCULATION &

ACCESS

This chapter addresses the responses to the emerging issues at the

reserve, particularly those of access and circulation in the light of

opportunities created by the purchase of the new property and existing

constraints around the rock.

Other scenarios were explored but were not considered appropriate.

Major Events Parking

34

Existing public road system used as the parks

circulation system to access the new property.

Avoids construction of costly roads through the site

although would require money collection at

separate carparking areas.

Possible retain as race day only entry.

Possible picnic area/carpark off Straws Lane.

Main controlled access at existing South Rock

Road entry.

Adoption of Boom Gates at entries may avoid

internal roading.

Access &

Circulation

35

The current north-south road connection

severely cramps the space between the

racecourse buildings and the eastern flank of

"the Rock". This connection shold be broken,

with the road downgraded to a narrowed

walking track/emergency vehicle access.

Indigenous planting would be used to screen

racetrack buildings and to encroach down from

"The Rock" to soften the experience.

Road Closure

Behind Race

Enclosure

36

One-way loop track entering along toe of slope and exiting

beside creek would solve the problem of the current road being

too close to the base of the rock.

S.W. Picnic Area

37

Picnic area developed on the hill in the new

property. The major initial goal is to re-vegetate

this area in order to introduce picnicking as soon

as possible. Cars would be located in trees on the

side of the hill, reducing visual intrusion to view

from "The Rock".

Eastern

Picnic Area

38

views over track from elevated picnic positions

Race Day Viewing

39

controlled single entry to rock walk through

interpretive centre

multiple entries to rock walk would help to

spread visitor loads away from the stairway,

but would be difficult to control

Access to Rock

40

11. REVENUE COLLECTION

AND ACCESS

5 options for revenue collection and access were considered and are

included in the appendix. The preferred option is a modification of

one of these and is outlined below.

11.1

REVENUE COLLECTION AT OUTER PERIMETER

— PREFERRED OPTION

The need to collect an entry fee from visitors greatly affects the

management as well as access and circulation options for the reserve.

An entry fee to access the reserve means that fencing is required

around the entire reserve and money collection points need to be

positioned at every entry. Given the current practice of manning the

gates, this expensive and labour intensive process, potentially limits

the number of entries, increasing internal road costs.

It is possible to make use of access control systems such as

multiple-coin operated boom-gates at the entries, freeing-up staff to go

about other tasks. This system has been applied successfully at the

You Yangs State Park. The all-up cost of a multi-denominational coin

collector and associated entry road works comes to approximately $12

— $15,000, an economically realistic alternative to a rangers wage

spent doing the same thing.

The preferred option proposes retaining the existing 2 entries, but

installing a boom gate at the southern entry. The northern road and

access gate would remain as is, servicing the picnic area to the

north-east of "The Rock" on race days only. A new connection road

could link the eastern riverflat picnic area to the picnic areas on the

hill of the new property in the future as the area is developed.

Boom Gates (including ass. road works)

$15,000 each x 1

Riverflat to Hill access road

650 linear metres x $120/m

Estimated total

$15,000.00

$78,000.00

$93,000.00

41

11.2

ALTERNATIVE OPTION

This option proposes 2 entries, one from the south - (for which no new

costing has been calculated as it already exists), accessing the main

picnic area, and a new road coming in from the east off Straws Lane.

This new entry point would access the new picnic area on "the hill"

and could link through the new property to the existing main picnic

areas.

Boom Gates (including ass. road works)

$15,000 each x 2

New carpark for 50 cars at picnic area on

"the hill" and access roading

Optional access road from "the hill" to the

existing main picnic area

650 linear metres x $120/m

Estimated total

$30,000.00

$30,000.00

$78,000.00

$138,000.00

42